DATA LOGGING

5.1  Data Logging Overview

The output data from the On-Board Dyno may be recorded by a PC

type  computer. Typically the computer will be a battery operated

laptop.  The  On-Board  Dyno outputs  the  values  of Horsepower,

Acceleration,  and  Speed  in a  RS232/binary/packet  format. The

output  rate is 25 packets per second. Software serves to collect

the  binary data  under a  user specified  name. After  a test is

completed,  additional  software  serves  to  convert  the binary

format to an ASCII, comma delimited, spreadsheet format. The data

is  converted to a user  named series of 60  second long files to

ease data searching and analysis.

5.2  Software Installation

From  the  supplied floppy  disk, make  a  backup copy  of the

DL.exe  and DP.exe files.  Make a subdirectory  called DYNOLOG on

the  hard drive of the data logging computer. Copy the DL.exe and

DP.exe  files to the  DYNOLOG subdirectory. It  is also suggested

that  DYNOLOG be placed in the path so that it may be called from

other locations such as a Ram Drive.

5.3  Ram Drive

Although  data may be  logged directly to  the hard drive, the

vibration  of the  vehicle may  disturb the  hard drive  and it's

ability  to record the  data. Hard drives are  also not known for

their  speed and their power consumption will shorten the battery

life of the data logging computer. It is therefore suggested that

a  ram drive be  created if there is  sufficient ram available to

support such a drive. A 1MB ram drive will support about one hour

of  data logging. You should be able to find the instructions for

the creation of a ram drive in your DOS manual.

5.4  Connections

The  On-Board Dyno is typically connected to a laptop computer

with the supplied phono plug to 9 pin D-Sub connector cable. Data

is  transmitted to pin 2 of the  9 pin D-Sub connector with pin 5

being signal ground. In some cases it may be necessary to use a 9

pin to 25 pin adapter.

5.5  Data Logging

When the test is about to begin, turn on the On-Board Dyno and

the  computer.  Log  onto  the ram  drive  or  enter  the DYNOLOG

subdirectory. Type DL.exe and a name with extension for the test.

Name and extension must be of a DOS format.

Example: DL.exe NEWTEST.log.

Press ENTER to start data logging.

Press Esc to terminate the data logging.

To  prevent loss of data when using  ram drive to log data, it is

suggested that the test file(s) be saved to the hard drive before

turning the computer off.

5.6  Data Conversion

To  convert data to a spreadsheet  format, type DP.exe and the

name of the test file, new name for converted file, the word SKIP

if you want to skip data when the vehicle was not moving, and the

number  of milli seconds over which you would like to average the

data.

Example: DP.exe NEWTEST.log NEWCONV.f01 SKIP 1000

It  is suggested  that only  the values  of 40,  200, 1000, 2000,

5000,  or  10,000 be  used  for the  number  of milli  seconds to

average the data.

Press ENTER to begin the conversion process.

A  series  of 60  second files  will  be created  with extensions

beginning with f01.

Example: NEWCONV.f01, NEWCONV.f02, NEWCONV.f03 ---- etc.

5.7  Spreadsheet

Import   the  desired  converted   files  to  the  spreadsheet

application of you choice using the comma/quote delimited format.

Once  the data is imported to  the spreadsheet you should be able

to view, graph, and analyze your data with ease.